MODULE 2 ASSIGNMENT

1. Brief about SplitMAC architecture and how it improves the AP's performance

Split MAC architecture is used in Lightweight Access Points (LWAPs) where the MAC functions are divided between the Access Point (AP) and the WLAN Controller (WLC).

- Lower MAC (on AP):
Handles real-time wireless operations such as:

  - Client association

  - Beacon/probe transmission

  - Frame acknowledgment (ACK)

  - Encryption/decryption

  - MU-MIMO / OFDMA operations

- Upper MAC (on WLC):
Centralized at the controller and handles:

  - Authentication & security

  - QoS management

  - Radio Resource Management (RRM)

  - Load balancing

  - Mobility & roaming control

  - Traffic shaping and policy enforcement

How it improves AP performance

- Reduced Load on AP

  - Heavy processing tasks are shifted to the controller

  - AP hardware can be simpler and more efficient

- Centralized Management

  - All configurations are handled in one place (WLC)

  - No need to configure each AP individually

- Better Scalability

  - Supports large networks with hundreds/thousands of APs

  - Easy to add/remove APs

- Improved Network Efficiency

  - Intelligent decisions (RRM, load balancing) are centrally optimized

  - Ensures better channel usage and reduced interference

- Enhanced Security

  - Security policies are centrally enforced

  - Consistent authentication across all APs

- Faster Roaming

  - Controller manages mobility

2. Describe about CAPWAP, explain the flow between AP and Controller

CAPWAP (Control And Provisioning of Wireless Access Points) is a protocol used for communication between Lightweight Access Points (APs) and the WLAN Controller (WLC). It enables centralized management of APs.

- Used in controller-based architectures

- Helps in configuration, control, and data forwarding

- Establishes communication using secure tunnels

Flow between AP and Controller:

- IP Address Assignment (DHCP)

  - AP boots and requests IP address

  - DHCP process: Discover → Offer → Request → Acknowledge

- Controller Discovery

  - AP searches for available WLC

  - Sends Discovery Request

  - Receives Discovery Response

- DTLS Connection Establishment

  - Secure channel is created between AP and WLC

  - Ensures encrypted communication

- Join Process

  - AP sends Join Request

  - WLC responds with Join Response

  - AP becomes part of the controller network

- Image Download (if required)

  - WLC may push firmware/software image to AP

  - Ensures compatibility

- Configuration Phase

  - AP sends Configuration Status Request

  - WLC sends configuration parameters (SSID, policies, etc.)

- Run State (Normal Operation)

  - AP starts serving clients

  - Data and control traffic flow via CAPWAP tunnels

- Tunnel Maintenance

  - Keepalive messages exchanged

  - Echo Request / Echo Response ensures connectivity

- Configuration Updates

  - Any changes from WLC are pushed dynamically to AP

3. Where this CAPWAP fits in OSI model, what are the two tunnels in CAPWAP and its purpose

CAPWAP operates over the Transport Layer (Layer 4) of the OSI model.

- Uses UDP for communication between AP and WLC

- Encapsulates wireless data and control messages

- Works above IP (Layer 3) and below application-level processing

So, CAPWAP mainly fits in:

- Layer 4 (Transport Layer) – UDP-based communication

- Supports functions interacting with Layer 2 (MAC) operations

Two Tunnels in CAPWAP

CAPWAP establishes two tunnels between AP and WLC:

1. Control Tunnel

- Used for management and control communication

- Carries:

  - Configuration data

  - Authentication information

  - AP management messages

- Fully encrypted (DTLS) for security

Purpose:

- To manage and control APs centrally

- Ensures secure communication between AP and controller

2. Data Tunnel

- Used for actual user data traffic

- Carries:

  - Client data (Wi-Fi traffic)

  - Frames from AP to controller

Purpose:

- To forward client traffic through the controller

- Enables centralized processing like QoS, security, and policy enforcement

4. What is the difference between Lightweight APs and Cloud-based APs

| Feature | Lightweight APs (LWAP) | Cloud-based APs |
| --- | --- | --- |
| Architecture | Uses Split MAC with centralized WLAN Controller (WLC) | Autonomous APs managed via Cloud Controller |
| Controller Location | On-premise (physical/virtual WLC) | Hosted in the cloud |
| Management | Fully controlled by WLC | Centrally managed through cloud dashboard |
| Data Traffic Flow | Both control and data traffic go through WLC (via CAPWAP) | Only control traffic goes to cloud; data stays local |
| Dependency | AP depends heavily on WLC for operation | AP works independently for data; cloud used for management |
| Deployment Type | Ideal for large centralized networks (campus, enterprise) | Ideal for distributed networks (multiple branch locations) |
| Scalability | Scales with controller capacity | Highly scalable via cloud infrastructure |
| Configuration | Done via WLC | Done via cloud interface |
| Analytics & Monitoring | Limited to controller capabilities | Advanced analytics, AI/ML insights available |
| Failure Impact | If WLC fails → AP functionality affected | If cloud fails → AP still forwards data locally |

5. How the CAPWAP tunnel is maintained between AP and controller

The CAPWAP tunnel between the Access Point (AP) and the WLAN Controller (WLC) is maintained using continuous monitoring and keepalive mechanisms.

CAPWAP Tunnel Maintenance Process

- Keepalive Messages

  - AP and WLC periodically exchange keepalive signals

  - Ensures that both devices are still reachable

- Echo Request / Echo Response

  - AP sends Echo Request to WLC

  - WLC replies with Echo Response

  - Confirms the tunnel is active

- DTLS Session Maintenance

  - Control tunnel is secured using DTLS encryption

  - Session is continuously maintained for secure communication

- Configuration Synchronization

  - WLC regularly pushes updates to AP

  - Ensures AP configuration stays consistent

- Heartbeat Monitoring

  - If responses are not received within a time limit → connection is considered lost

Failure Handling

- If CAPWAP tunnel fails:

  - AP tries to re-establish connection with WLC

  - May restart discovery and join process

  - In some modes (like FlexConnect), AP continues limited operation

6. What is the difference between Sniffer and monitor mode, use case for each mode

| Feature | Sniffer Mode | Monitor Mode |
| --- | --- | --- |
| Definition | AP captures wireless packets and sends them to WLC for analysis | AP continuously scans the RF environment for network information |
| BSS (Client Service) | Does NOT provide BSS (no client connectivity) | Does NOT provide BSS (no client connectivity) |
| Main Function | Packet capture (like a wireless packet analyzer) | Network monitoring and surveillance |
| Data Handling | Captures raw packets and forwards to controller | Collects RF data, rogue AP info, interference details |
| Purpose | Troubleshooting and deep packet inspection | Security monitoring and network optimization |
| Focus Area | Traffic-level analysis | RF/environment-level analysis |
| Output | Packet-level data for debugging | Alerts, telemetry, rogue detection info |

7. If WLC deployed in WAN, which AP mode is best for local network and how?

Best AP Mode: FlexConnect Mode

FlexConnect mode is the most suitable when the WLC is deployed in WAN (remote location).

Reason:

- In Local mode:

  - Both control and data traffic go through WLC

  - Causes high latency if WLC is far (WAN)

- In FlexConnect mode:

  - AP can locally switch data traffic

  - Only control traffic goes to WLC

How FlexConnect Works:

- Under normal conditions:

  - AP connects to WLC via CAPWAP

  - Receives configuration from WLC

- During operation:

  - Client data is forwarded locally (not through WAN)

  - Reduces delay and WAN dependency

- If WLC connection is lost:

  - AP switches to autonomous-like behavior

  - Continues to serve clients locally

Advantages:

- Reduced latency (no WAN backhaul for data)

- Better performance for local users

- WAN bandwidth saving

- Resilience (works even if WLC is down)

8. What are challenges if deploying autonomous APs (more than 50) in large network like university

Challenges

- Manual Configuration

  - Each AP must be configured individually (SSH/Web/Console)

  - Time-consuming and error-prone

- No Centralized Management

  - No single controller to manage all APs

  - Difficult to monitor and control the network

- RF Management Issues

  - Channel and power settings must be manually configured

  - Leads to interference and poor performance

- Difficult Scalability

  - Managing 50+ APs becomes complex

  - Adding new APs increases administrative overhead

- Firmware Upgrade Complexity

  - Updates must be done individually on each AP

  - High maintenance effort

- Security Management

  - Security policies need to be configured per AP

  - Risk of inconsistent configurations

- VLAN Complexity

  - VLANs must span across the entire network

  - Creates large broadcast domains (bad practice)

- No Load Balancing / Optimization

  - No centralized RRM or load balancing

  - Uneven client distribution

9. What happens on wireless client connected to Lightweight AP in local mode if WLC goes down.

In Local mode, both control and data traffic from the AP are sent to the WLAN Controller (WLC) through the CAPWAP tunnel.

What Happens When WLC Goes Down:

- CAPWAP Tunnel Failure

  - Connection between AP and WLC is lost

- Client Connectivity Impact

  - Since data traffic depends on WLC,
  Clients lose network/internet access

- No New Client Associations

  - New devices cannot connect to the AP

- Existing Clients

  - Already connected clients will get disconnected or experience network failure

- AP Behavior

  - AP keeps trying to reconnect to WLC

  - Cannot function fully without controller

Key Reason

- In Local mode:

  - All traffic (control + data) → WLC

  - Without WLC → AP cannot forward data