MODULE 3 ASSIGNMENT

1. What are the different 802.11 PHY layer standards? Compare their characteristics.

The Physical (PHY) layer in Wi-Fi defines how data is transmitted over the air (frequency, modulation, data rate, etc.).

| Standard | Frequency Band | Max Data Rate | Modulation | Channel Width | Key Features |
| --- | --- | --- | --- | --- | --- |
| 802.11b | 2.4 GHz | 11 Mbps | DSSS | 20 MHz | Early Wi-Fi, long range, high interference |
| 802.11a | 5 GHz | 54 Mbps | OFDM | 20 MHz | Less interference, shorter range |
| 802.11g | 2.4 GHz | 54 Mbps | OFDM | 20 MHz | Backward compatible with 11b |
| 802.11n (Wi-Fi 4) | 2.4 & 5 GHz | Up to 600 Mbps | OFDM | 20/40 MHz | Introduced MIMO |
| 802.11ac (Wi-Fi 5) | 5 GHz | Up to ~6.9 Gbps | OFDM | 20/40/80/160 MHz | MU-MIMO, higher throughput |
| 802.11ax (Wi-Fi 6) | 2.4 & 5 GHz | Up to ~9.6 Gbps | OFDMA | 20–160 MHz | OFDMA, better efficiency |
| 802.11ax (Wi-Fi 6E) | 6 GHz | Up to ~9.6 Gbps | OFDMA | 20–160 MHz | Less congestion, more channels |
| 802.11be (Wi-Fi 7) | 2.4, 5, 6 GHz | Up to ~46 Gbps | OFDMA + 4K QAM | Up to 320 MHz | Multi-Link Operation, ultra high speed |

2. What are DSSS and FHSS? How do they work?

DSSS (Direct Sequence Spread Spectrum)

DSSS is a spread spectrum technique used in early Wi-Fi (802.11b) where the signal is spread over a wider frequency band.

How DSSS Works

- Original data bits are combined with a pseudo-random spreading code (chip sequence)

- Each bit is converted into multiple smaller bits called chips

- These chips are transmitted over a wider bandwidth

- At the receiver, the same code is used to reconstruct the original data

Key Features

- High resistance to noise and interference

- More reliable communication

- Used in 802.11b (2.4 GHz)

FHSS (Frequency Hopping Spread Spectrum)

FHSS is a technique where the signal rapidly switches between different frequency channels in a predefined pattern.

How FHSS Works

- Data is divided into small chunks

- Each chunk is transmitted on a different frequency channel

- Transmitter and receiver follow the same hopping sequence

- If interference occurs on one frequency, communication continues on others

Key Features

- High security (difficult to intercept)

- Avoids interference by hopping

- Used in early wireless systems (less common in modern Wi-Fi)

3. How do modulation schemes work in the PHY layer? Compare different modulation schemes and their performance across various Wi-Fi standards.

In the PHY layer, modulation is used to convert digital data (bits) into radio signals that can be transmitted over the air.

- Data bits are mapped to symbols

- Symbols are represented using:

  - Amplitude (signal strength)

  - Phase (signal angle)

  - Frequency (in some cases)

- More bits per symbol → higher data rate, but requires better signal quality

| Modulation | Bits per Symbol | Used In | Performance |
| --- | --- | --- | --- |
| BPSK | 1 bit | 802.11a/g/n | Very robust, low speed |
| QPSK | 2 bits | 802.11a/g/n | Moderate speed and reliability |
| 16-QAM | 4 bits | 802.11a/g/n/ac | Higher speed, moderate SNR needed |
| 64-QAM | 6 bits | 802.11n/ac | High speed, needs good signal |
| 256-QAM | 8 bits | 802.11ac | Very high speed, sensitive to noise |
| 1024-QAM | 10 bits | 802.11ax (Wi-Fi 6) | Ultra high speed, requires excellent signal |
| 4096-QAM (4K-QAM) | 12 bits | 802.11be (Wi-Fi 7) | Extremely high speed, very sensitive |

Performance Comparison Across Wi-Fi Standards

| Standard | Modulation Used | Max Efficiency | Signal Requirement |
| --- | --- | --- | --- |
| 802.11b | DSSS (not QAM-based) | Low | Works in weak signals |
| 802.11a/g | Up to 64-QAM | Medium | Moderate signal quality |
| 802.11n | Up to 64-QAM | High | Good signal needed |
| 802.11ac | Up to 256-QAM | Very High | Strong signal required |
| 802.11ax | Up to 1024-QAM | Ultra High | Very strong signal |
| 802.11be | Up to 4096-QAM | Extremely High | Excellent signal only |

4. What is the significance of OFDM in WLAN? How does it improve performance?

OFDM is a transmission technique used in modern Wi-Fi standards (802.11a/g/n/ac/ax) where a high-speed data stream is divided into multiple lower-speed streams and transmitted simultaneously over different subcarriers.

How OFDM Works:

- The available channel is divided into many small subcarriers

- Data is split into parallel streams

- Each stream is transmitted on a separate subcarrier

- Subcarriers are orthogonal → no interference between them

- At the receiver, signals are combined to reconstruct the original data

Significance of OFDM:

- Enables high data rates

- Efficient use of available spectrum

- Supports advanced features like MIMO and high-order modulation

How OFDM Improves Performance

- Higher Throughput

  - Parallel transmission increases total data rate

- Reduced Interference

  - Orthogonal subcarriers avoid overlapping interference

- Better Resistance to Multipath Fading

  - Splitting data into smaller streams reduces signal distortion

- Efficient Spectrum Usage

  - Subcarriers are closely spaced but do not interfere

- Supports High Modulation (QAM)

  - Works well with 64-QAM, 256-QAM, etc.

Advantages:

- Many Paths: It sends data over many small channels at once instead of just one big one.

- Faster: It can carry much more information in less time.

- Stays Clear: It is much better at ignored "noise" or electronic interference.

- Smarter: It uses the available wireless space more effectively without wasting it.

5. How are frequency bands divided for Wi-Fi? Explain different bands and their channels.

Wi-Fi operates in three main frequency bands:

- 2.4 GHz band

- 5 GHz band

- 6 GHz band (Wi-Fi 6E / Wi-Fi 7)

Each band is divided into multiple channels used for communication.

1. 2.4 GHz Band

- Frequency range: 2.4 – 2.4835 GHz

- Total channels: 14 (varies by country)

- Common usable non-overlapping channels: 1, 6, 11

Characteristics

- Longer range

- Better wall penetration

- High interference (Bluetooth, microwave, etc.)

- Lower data rates

2. 5 GHz Band

- Frequency range: 5.15 – 5.825 GHz

- Many channels (varies by region)

- Supports wider channels: 20 / 40 / 80 / 160 MHz

Characteristics

- Higher data rates

- Less interference than 2.4 GHz

- Shorter range

- More non-overlapping channels

3. 6 GHz Band (Wi-Fi 6E / Wi-Fi 7)

- Frequency range: 5.925 – 7.125 GHz

- Large number of channels available

- Supports wide channels (up to 320 MHz)

Characteristics

- Very high speed

- Minimal interference (new band)

- Shortest range

- Ideal for high-density environments

Channel Comparison:

| Feature | 2.4 GHz | 5 GHz | 6 GHz |
| --- | --- | --- | --- |
| Range | Long | Medium | Short |
| Speed | Low | High | Very High |
| Interference | High | Medium | Very Low |
| Channels | Few (3 non-overlapping) | Many | Very Many |
| Channel Width | 20 MHz | Up to 160 MHz | Up to 320 MHz |

6. What is the role of Guard Intervals in WLAN transmission? How does a short Guard Interval improve efficiency?

A Guard Interval is a small time gap inserted between transmitted symbols in OFDM-based Wi-Fi systems.

Role of Guard Interval:

- Prevents Inter-Symbol Interference (ISI)

  - In wireless communication, signals may reflect and arrive late (multipath)

  - GI ensures delayed signals do not interfere with the next symbol

- Improves Signal Reliability

  - Allows receiver to correctly distinguish symbols

  - Reduces errors due to echo and reflections

Types of Guard Intervals:

- Normal GI (800 ns): The slower, more reliable default setting that prevents interference.

- Short GI (400 ns): A faster setting that boosts data speeds but is more prone to errors.

How Short Guard Interval Improves Efficiency:

- Reduces Overhead

  - Less time wasted between symbols

- Increases Data Rate

  - More symbols can be transmitted per second

- Improves Throughput (~10–11%)

  - Especially beneficial in good signal conditions

Trade-Off:

- Short GI requires:

  - Good signal quality

  - Less multipath interference

- In poor conditions:

  - Normal GI is preferred for reliability

7. Describe the structure of an 802.11 PHY layer frame. What are its key components?

In WLAN, the PHY layer frame is called a PPDU (PLCP Protocol Data Unit).
It consists of two main parts:

- Preamble

- PLCP Header + Payload

Key Components:

1. Preamble

- Sent at the beginning of the frame

- Used for:

  - Synchronization

  - Signal detection

  - Channel estimation

- Helps the receiver prepare to decode the incoming data

2. PLCP Header (Physical Layer Convergence Protocol Header)

- Contains transmission information such as:

  - Data rate

  - Frame length

  - Modulation type

- Tells the receiver how to interpret the payload

3. PSDU (Payload Data)

- Actual data coming from MAC layer

- Contains:

  - MAC frame

  - User data

Additional PHY Components

- PMD (Physical Medium Dependent)

  - Handles actual signal transmission over air

  - Includes modulation and radio transmission

8. What is the difference between OFDM and OFDMA?

| Feature | OFDM (Orthogonal Frequency Division Multiplexing) | OFDMA (Orthogonal Frequency Division Multiple Access) |
| --- | --- | --- |
| Basic Concept | Splits channel into multiple subcarriers for a single user | Splits channel into subcarriers and allocates them to multiple users simultaneously |
| User Access | One user uses all subcarriers at a time | Multiple users share subcarriers at the same time |
| Efficiency | Less efficient in multi-user environments | Highly efficient in dense networks |
| Latency | Higher (users wait for their turn) | Lower (simultaneous transmission) |
| Throughput | Good for single-user transmission | Better overall throughput in multi-user scenarios |
| Resource Allocation | Fixed allocation | Dynamic allocation (Resource Units - RUs) |
| Wi-Fi Standards | Used in 802.11a/g/n/ac | Introduced in 802.11ax (Wi-Fi 6) |
| Use Case | Best for fewer devices | Best for many devices (IoT, crowded networks) |

9. What is the difference between MIMO and MU-MIMO?

| Feature | MIMO (Multiple Input Multiple Output) | MU-MIMO (Multi-User MIMO) |
| --- | --- | --- |
| Basic Concept | Uses multiple antennas to transmit data to one user at a time | Uses multiple antennas to transmit data to multiple users simultaneously |
| User Handling | Single-user communication per transmission | Multi-user communication in parallel |
| Efficiency | Improves speed for one device | Improves overall network efficiency |
| Throughput | High for a single user | Higher total throughput across multiple users |
| Latency | Other users must wait | Reduced latency (parallel transmission) |
| Wi-Fi Standards | Introduced in 802.11n (Wi-Fi 4) | Introduced in 802.11ac (Wi-Fi 5), enhanced in Wi-Fi 6 |
| Direction Support | Uplink and Downlink (varies by standard) | Initially Downlink only, later both UL & DL in Wi-Fi 6 |
| Best Use Case | Few devices, high-speed need | Many devices in dense environments |

10. What are PPDU, PLCP, and PMD in the PHY layer?

PPDU:

- Physical Protocol Data Unit

- Complete PHY layer frame transmitted over the air

- Final frame sent over the wireless medium

- Contains:

  - Preamble

  - PLCP header

  - PSDU (MAC data)

PLCP:

- Physical Layer Convergence Protocol

- Interface between MAC and PHY; prepares data for transmission

- Converts MAC layer data into PHY format

- Adds header information (rate, length, etc.)

- Ensures compatibility between MAC and PHY

PMD:

- Physical Medium Dependent

- Handles actual signal transmission (radio, modulation, frequency)

- Responsible for physical transmission

- Includes:

  - Modulation (QAM, OFDM)

  - Frequency selection

  - Signal transmission over air

11. What are the types of PPDU? Explain the PPDU frame format across different Wi-Fi generations.

Types of PPDU:

| PPDU Type | Wi-Fi Standard | Description |
| --- | --- | --- |
| Legacy PPDU | 802.11a/b/g | Basic format for backward compatibility |
| HT PPDU | 802.11n (Wi-Fi 4) | Supports MIMO and higher throughput |
| VHT PPDU | 802.11ac (Wi-Fi 5) | Supports wider channels and MU-MIMO |
| HE PPDU | 802.11ax (Wi-Fi 6) | Supports OFDMA and high efficiency |
| EHT PPDU | 802.11be (Wi-Fi 7) | Supports ultra-high speed and multi-link |

PPDU Frame Format Across Generations:

| Component | Legacy | HT (Wi-Fi 4) | VHT (Wi-Fi 5) | HE (Wi-Fi 6) | EHT (Wi-Fi 7) |
| --- | --- | --- | --- | --- | --- |
| Preamble | Basic preamble | Enhanced (HT-STF, HT-LTF) | VHT preamble fields | HE-specific fields | Advanced multi-link preamble |
| Header | Basic PLCP header | HT-SIG | VHT-SIG | HE-SIG | EHT-SIG |
| Payload (PSDU) | MAC data | MAC data | MAC data | MAC data | MAC data |
| Key Features | Simple transmission | MIMO support | MU-MIMO, wider bandwidth | OFDMA, better efficiency | 320 MHz, 4K QAM, multi-link |

12. How is the data rate calculated?

The data rate (throughput) in Wi-Fi depends on multiple PHY layer parameters such as bandwidth, modulation, coding, and spatial streams.

Data Rate=Number of Bits per Symbol×Number of Subcarriers×Coding Rate×Number of Spatial StreamsSymbol Duration

| Parameter | Description |
| --- | --- |
| Bits per Symbol | Depends on modulation (e.g., BPSK = 1, QPSK = 2, 64-QAM = 6, 1024-QAM = 10) |
| Subcarriers | Number of OFDM subcarriers used for data transmission |
| Coding Rate | Error correction efficiency (e.g., 1/2, 3/4, 5/6) |
| Spatial Streams | Number of parallel data streams (MIMO) |
| Symbol Duration | Time taken to transmit one symbol (depends on Guard Interval) |

Data rate increases when:

- Higher modulation is used (e.g., 1024-QAM > 64-QAM)

- Channel width increases (20 MHz → 160 MHz)

- More spatial streams are used (MIMO)

- Short Guard Interval is used

Factors Affecting data rate:

- Higher modulation → higher speed

- Wider channel → more data

- More streams → parallel transmission

- Short GI → faster transmission