MODULE 1 ASSIGNMENTS

1. In which OSL layer the Wi-Fi standard/protocol fits.

Wi-Fi operates mainly in the OSI Layer 1 and Layer 2:

- Layer 1 – Physical Layer (PHY):

Handles transmission of raw bits over the air.

Defines frequencies (2.4 GHz, 5 GHz, 6 GHz), modulation techniques (OFDM, DSSS), data rates, etc.

- Layer 2 – Data Link Layer (MAC sublayer):

Responsible for:

- Frame formatting

- MAC addressing

- Error detection

- Medium access control (CSMA/CA)

Ensures reliable communication between devices on the same network.

2. Can you share the Wi-Fi devices that you are using day to day life, share that device's wireless capability/properties after connecting to network. Match your device to corresponding Wi-Fi Generations based on properties

- Android Phone

Observed Wi-Fi Properties (after connecting):

Frequency Band: 2.4 GHz / 5 GHz

Link Speed: ~300–1200 Mbps (varies)

Channel Width: 20/40/80 MHz

Security: WPA2/WPA3

MIMO Support: Yes (2x2)

Wi-Fi Generation Mapping:

Supports Wi-Fi 5 (802.11ac) or Wi-Fi 6 (802.11ax)

- Laptop

Observed Wi-Fi Properties:

Frequency Band: 2.4 GHz / 5 GHz

Link Speed: ~433 Mbps to 2.4 Gbps

Channel Width: Up to 80/160 MHz

Security: WPA2/WPA3

Advanced Features: OFDMA, MU-MIMO (in newer laptops)

Wi-Fi Generation Mapping:

Typically Wi-Fi 5 (802.11ac) or Wi-Fi 6 (802.11ax)

3. What is BSS and ESS?

- BSS (Basic Service Set)

A Basic Service Set (BSS) is the smallest unit of a Wi-Fi network.

It consists of:

One Access Point (AP)

Connected wireless devices (stations/clients)

All devices communicate through the Access Point.

Each BSS is identified by a:   BSSID → usually the MAC address of the Access Point

Example:

Home Wi-Fi (one router + your phone, laptop) = 1 BSS

Types of BSS:

Infrastructure BSS: Uses an Access Point (most common)

Independent BSS (IBSS): No AP (device-to-device / ad-hoc)

- ESS (Extended Service Set)

An Extended Service Set (ESS) is formed by multiple BSSs connected together.

It consists of:

Multiple Access Points

Connected via a wired or wireless backbone

All APs share the same SSID (network name)

Purpose:

Provides larger coverage area

Allows seamless roaming between APs

Example:

Wi-Fi network in a college, office, or large building with many routers = ESS

4. What are the basic functionalities of Wi-Fi Accesspoint

- Providing Wireless Connectivity

Allows devices (phones, laptops, etc.) to connect using Wi-Fi

Broadcasts SSID (network name)

Operates on frequency bands like 2.4 GHz / 5 GHz

- Medium Access Control (MAC Function)

Uses CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance)

Ensures multiple devices share the wireless medium efficiently

Controls when a device can transmit data

- Authentication & Security

Verifies users before allowing access

Supports:

WPA2 / WPA3 security

Password-based or enterprise authentication

Encrypts data to protect communication

- Data Forwarding (Bridging)

Acts as a bridge between wireless and wired networks

Forwards data:

From Wi-Fi device → Router/Internet

From Internet → Wi-Fi device

- Device Association & Management

Handles:

Association (joining network)

Disassociation (leaving network)

Maintains list of connected devices

- Roaming Support

Helps devices switch between APs in a large network (ESS)

Ensures minimal connection drop

- Quality of Service (QoS)

Prioritizes traffic (e.g., video calls over downloads)

Improves performance for real-time applications

- Signal Coverage & Range Management

Controls transmission power

Extends wireless coverage area

5. Difference between Bridge mode and Repeater mode

| Feature | Bridge Mode | Repeater Mode |
| --- | --- | --- |
| Definition | Connects two networks using a wired/wireless link | Extends existing Wi-Fi coverage wirelessly |
| Main Purpose | Network extension (LAN to LAN connection) | Signal extension (increase Wi-Fi range) |
| Connection Type | Usually wired (Ethernet) or point-to-point wireless | Fully wireless (no cable required) |
| SSID | Same or different (depends on config) | Usually same SSID as main network |
| Performance | No significant speed loss | Speed reduces (typically ~50%) |
| Latency | Low | Higher due to signal repeating |
| Use Case | Connecting two buildings/networks | Extending Wi-Fi in home/office dead zones |
| Network Layer Role | Works like a bridge at Layer 2 | Works as signal repeater/amplifier |
| Configuration | Slightly complex | Easy to configure |
| Internet Source | From main router via wired/wireless backhaul | From existing Wi-Fi signal |

6. What are the differences between 802.11a and 802.11b.

| Feature | 802.11a | 802.11b |
| --- | --- | --- |
| Frequency Band | 5 GHz | 2.4 GHz |
| Maximum Speed | Up to 54 Mbps | Up to 11 Mbps |
| Range | Shorter | Longer |
| Signal Penetration | Poor (less wall penetration) | Better (can pass through walls) |
| Interference | Less interference | More interference (crowded band) |
| Modulation | OFDM (Orthogonal Frequency Division Multiplexing) | DSSS (Direct Sequence Spread Spectrum) |
| Cost | More expensive | Cheaper |
| Number of Channels | More non-overlapping channels | Fewer channels |
| Usage | Less common (earlier tech, replaced later) | Widely used earlier in home networks |

7. Configure your modem/hotspot to operate only in 2.4Ghz and connect your laptop/Wi-Fi device and capture the capability/properties in your Wi-Fi device. Repeat the same in 5Ghz and tabulate all the differences you observed during this

| Feature | 2.4 GHz Band | 5 GHz Band |
| --- | --- | --- |
| Frequency | 2.4 GHz | 5 GHz |
| Link Speed | Lower (e.g., 50–150 Mbps) | Higher (e.g., 200–800 Mbps) |
| Range | Longer range | Shorter range |
| Wall Penetration | Better | Poorer |
| Interference | High (Bluetooth, microwave, etc.) | Low |
| Channel Width | 20 / 40 MHz | 20 / 40 / 80 / 160 MHz |
| Stability | More stable over distance | Less stable far from router |
| Latency | Slightly higher | Lower |
| Number of Channels | Limited | More channels available |
| Best Use Case | Browsing, IoT devices | Gaming, streaming, high-speed tasks |

8. What is the difference between IEEE and WFA

| Feature | IEEE (Institute of Electrical and Electronics Engineers) | WFA (Wi-Fi Alliance) |
| --- | --- | --- |
| Full Form | Institute of Electrical and Electronics Engineers | Wi-Fi Alliance |
| Type | Standards development organization | Industry certification organization |
| Main Role | Develops technical standards (e.g., 802.11) | Certifies products for interoperability |
| Focus Area | Defines how Wi-Fi technology works | Ensures devices work together properly |
| Output | Standards like 802.11a/b/g/n/ac/ax | Certifications like Wi-Fi 4, 5, 6, WPA3 |
| Membership | Engineers, researchers, professionals | Companies (Apple, Intel, Cisco, etc.) |
| Example Work | Created 802.11 protocol | Certifies devices as “Wi-Fi Certified” |
| User Interaction | Indirect (users don’t deal with IEEE directly) | Direct (seen as Wi-Fi logos on devices) |

9. List down the type of Wi-Fi internet connectivity backhaul, share your home/college's wireless internet connectivity backhaul name and its properties

Backhaul is the connection between your Wi-Fi router/access point and the internet service provider (ISP). It carries data from your local network to the internet.

| Type | Description |
| --- | --- |
| Fiber (FTTH) | Uses optical fiber; very high speed (100 Mbps to multi-Gbps), low latency |
| DSL (Telephone line) | Uses copper phone lines; moderate speed (up to ~100 Mbps) |
| Cable (DOCSIS) | Uses coaxial cable (TV line); high speed, widely used |
| Cellular (4G/5G) | Uses mobile network; portable (hotspots), speed depends on signal |
| Wireless Backhaul | Point-to-point wireless link (used in rural/remote areas) |
| Satellite | Uses satellite communication; works anywhere but high latency |

Home Network: Airtel Fiber

Backhaul Type: Fiber (FTTH - Fiber to the Home)

Properties:

Very high speed (100 Mbps to 1 Gbps or more)

Low latency

Stable connection

Suitable for streaming, gaming, video calls

College Network: BSNL (via Ethernet)

Backhaul Type: Fiber or DSL (depends on BSNL setup, mostly fiber in colleges)

Connection Type to Device: Wired Ethernet

Properties:

Stable and reliable

Lower latency than Wi-Fi

Consistent speed

Suitable for labs, downloads, and academic work

10. List down the Wi-Fi topologies and use cases of each one. give answer for each.

| Topology | Description | Use Case |
| --- | --- | --- |
| Infrastructure Mode (BSS) | Devices connect through a single Access Point | Home Wi-Fi, small offices |
| Extended Service Set (ESS) | Multiple APs connected to form a large network | Colleges, offices, campuses |
| Ad-hoc Mode (IBSS) | Devices connect directly without an AP | File sharing between laptops, temporary networks |
| Mesh Topology | Multiple APs (nodes) connect with each other wirelessly | Smart homes, large buildings, public Wi-Fi |
| Point-to-Point (P2P) | Direct wireless link between two devices/APs | Connecting two buildings |
| Point-to-Multipoint | One AP connects to multiple clients | ISPs providing wireless internet |